SECTION 030100 – MAINTENANCE OF CAST-IN-PLACE CONCRETE

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Repair of structural and non structural cracks 10 mil (0.010 inches) wide and wider in concrete slabs.

          - Gravity fed repair of concrete slabs cracks.

        - Related Sections:

          - Division 01 Section "General Requirements".

          - Division 01 Section "Special Procedures".

          - Division 07 Section "Pedestrian Traffic Coatings".

      - SUBMITTALS

        - Submit under provisions of Division 01 Section "General Requirements."

        - Product data for each type of product specified.

        - Notarized certification from the manufacturer attesting to the subcontractor’s Approved Contractor status.

        - Notarized certification from the manufacturer stating that the repair materials meet specification requirements.

      - QUALITY ASSURANCE

        - Comply with Division 01 Section "Special Procedures".

        - Manufacturers Qualifications: The manufacturer of the specified product shall have a program for training, certifying and technically supporting a national Approved Contractor Program for a minimum of  10 years. The manufacturer shall also be certified as meeting the ISO 9000 Quality Standard at all facilities producing the specified product.

        - Subcontractor Qualifications: Subcontractors shall be an Approved Contractor of the manufacturer of the specified product, who has completed a program of instruction in the use of the specified product.

      - DELIVERY, STORAGE AND HANDLING

        - Deliver the specified product in original, unopened containers with the manufacturer’s name, labels, product identification and batch numbers.

        - Store the specified product in accordance with the manufacturer’s recommendations.

      - PROJECT CONDITIONS

        - Environmental Conditions:  Do not apply material if it is raining, if rain is imminent or surfaces are wet.

        - Precautions shall be taken to avoid damage to surfaces near the work zone while handling, mixing or applying the product.

- PRODUCTS

      - MANUFACTURERS

        - Sikadur 55 SLV Healer/sealer as manufactured by Sika Corporation, Lyndhurst, New Jersey.

        - Substitutions: The use of a substitute product will be considered by the University provided that the Subcontractor requests the substitution in writing with documentation from an independent testing laboratory that the proposed substitute product meets or exceeds the performance criteria and has at least two years of successful performance on at least 5 installations.

      - PERFORMANCE CRITERIA

        - Properties of the mixed epoxy resin:

          - Pot life:	Approximately 25 minutes

          - Color:	Clear, amber

        - Properties of the cured epoxy resin:

          - Compressive strength when tested in accordance with ASTM D-695:

            - 1 day:		5000 psi (34.5 MPa) minimum

            - 3 days:	12000 psi (8.28 MPa), minimum

- EXECUTION

      - PREPARATION

        - Bead blast concrete surfaces to be epoxy injected.

        - Clean surfaces to be epoxy injected with pressurized air.

      - INSTALLATION

        - Apply epoxy by gravity feed in strict accordance with the manufacturer’s instructions.

        - Broadcast silica sand on the treated surface to refusal.

END OF SECTION 030100