SECTION 311000 – SITE CLEARING

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Removal and disposal of surface debris.

          - Removal and disposal of paving, curbs, gutters, fences, posts, or other structures indicated on the drawings.

          - Site clearing and disposal of plant life and grass.

          - Removal and disposal of trees and shrubs.

          - Removal and disposal of root system of trees and shrubs.

          - Topsoil excavation and stockpiling.

        - Related Sections:

          - Division 01 Section "General Requirements."

          - Division 01 Section "Special Procedures."

          - Division 31 Section "Backfill".

      - SUBMITTALS

        - Comply with the provisions of Division 01 Section "General Requirements".

        - Product Data:  For each type of product indicated.

          - Tree Paint:  Include product label and manufacturer's application instructions specific to the Project.

      - QUALITY ASSURANCE

        - Prior to drilling or excavation, obtain a "Permit to Penetrate or Excavate Surfaces at LBNL" in accordance with Division 01 Section "Special Procedures".  Remove and dispose of paving, curbs, gutters, fences, posts, and structures indicated for removal on the Drawings.

        - Conform to applicable codes and local, state and federal regulations for disposal of debris and use of herbicides.  Burning of debris, lumber or scrap will not be permitted.

        - Coordinate clearing work with the Project Manager.

        - Obtain the University's approval prior to removal of tree branches.  Removal of tree branches shall be under the supervision of the University.

- PRODUCTS

      - MATERIALS

        - Tree Paint:  Non-toxic, commercial grade, suitable for application on fresh cut branches.

- EXECUTION

      - TEMPORARY EROSION AND SEDIMENTATION CONTROL

        - Provide temporary erosion and sedimentation control measures to prevent soil erosion and discharge of soil-bearing water runoff or airborne dust to adjacent properties and walkways, according to [requirements of authorities having jurisdiction] [sediment and erosion control Drawings] [a sediment and erosion control plan, specific to the site, that complies with EPA 832/R-92-005 or requirements of authorities having jurisdiction, whichever is more stringent].

        - Inspect, repair, and maintain erosion and sedimentation control measures during construction until permanent vegetation has been established.

        - Remove erosion and sedimentation controls and restore and stabilize areas disturbed during removal.

      - PREPARATION

        - Verify that existing plant life designated to remain, is tagged or identified.

        - Sawcut paving, curbs gutters and other structures between portion to remain and portion to be removed.

      - PROTECTION

        - Locate, identify, and protect from damage utilities indicated to remain.

        - Protect trees, plant growth, and features indicated to remain.

        - Protect from damage or displacement bench marks and existing structures indicated to remain.

      - CLEARING

        - Clear areas required for access to site and execution of the Work.

        - Remove paving, curbs, and gutters, fences, posts, or structures indicated on the Drawings.

        - Remove trees and shrubs indicated.

          - In areas to be filled and under structures and roads, remove stumps, and main root system to a depth of not less than 2 feet (0.6 m) below the original ground surface.

          - Fill depressions made by grubbing with structural backfill to the original surface in accordance with Division 31 Section "Backfill" unless further excavation is required.

        - Clear undergrowth and dead wood without disturbing subsoil.

        - Apply herbicide approved by the University to remaining stumps to inhibit growth.

        - Cut tree branches in conflict with the construction close to the bole in a workmanlike manner.  Remove other tree branches in such a manner that the tree will present a balanced appearance.  Scars resulting from the removal of branches shall be treated with a heavy coat of tree paint.

        - Remove debris, rock, and extracted plant life from site.

      - TOPSOIL EXCAVATION

        - Excavate topsoil from areas to be further excavated, re-landscaped, or re-graded.

        - Stockpile in area designated on site to depth not exceeding 8 feet (2.5 m) and protect from erosion.  Remove excess topsoil not being reused from site.

END OF SECTION 311000