SECTION 312316 - EXCAVATION

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Excavation for buildings and other structural foundations.

          - Excavation for roads and walkways.

          - Excavation for site structures and utilities.

        - Related Sections:

          - Division 01 Section "General Requirements."

          - Division 01 Section "Environmental, Safety, and Health Procedures".

          - Division 01 Section "Special Procedures."

          - Division 31 Section "Site Clearing".

          - Division 31 Section "Backfilling".

          - Division 31 Section "Slope Protection ".

      - REFERENCES

        - General:

          - The following documents form part of the Specifications to the extent stated. Where differences exist between codes and standards, the one affording the greatest protection shall apply.

          - Unless otherwise noted, the referenced standard edition is the current one at the time of commencement of the Work.

          - Refer to Division 01 Section "General Requirements" for the list of applicable regulatory requirements.

        - 29CFR - OSHA Construction Standards:

          - Part 1926 - Safety and Health Regulations for Construction – Subpart P - Excavations

      - SUBMITTALS

        - Submit under provisions of Division 01 Section "General Requirements."

        - Shop Drawings:  Submit a construction drainage plan showing the collection and disposal of surface and subsurface water that may be encountered in the course of construction.

        - Shoring Plan:  Before excavating 5 feet or more in depth, submit a shoring plan in accordance with the requirements in Division 01 Section "General Requirements."

      - PROJECT CONDITIONS

        - Verify that survey benchmark and intended elevations for the Work are as indicated.

- PRODUCTS	NOT USED

- EXECUTION

      - PREPARATION

        - Underpin adjacent structures and roads which may be damaged by excavation work, including utilities and pipe chases.

        - Identify required lines, levels, contours, and datum.

        - Prior to penetrating any ground surface, obtain "Permit to Penetrate or Excavate Surfaces at LBNL" in accordance with Division 01 Section "Environmental, Safety, and Health Procedures".  Forty eight hours notice is normally required from when a permit is requested to when it is issued; the Subcontractor is responsible for scheduling the permit request to suit construction activities. Identify known underground, above ground, and aerial utilities.  Stake and flag locations.

        - Coordinate utility relocation or removal with the Project Manager.

        - Protect above and below grade utilities which are to remain.  Support exposed utilities as needed.

        - Protect plant life, lawns, and other site features indicated to remain.

        - Protect bench marks, existing structures, fences, sidewalks, paving, and curbs from excavation equipment and vehicular traffic.

      - EXCAVATION

        - Excavate subsoil required to accommodate building foundations, slabs-on-grade, paving, utilities and site structures, construction operations. The maximum slope inclinations shall comply with OSHA.

        - Excavate to working elevation[s] for piling  or caisson work.

        - Machine slope banks to angle of repose or less, unless shored in accordance with the references cited in Divisions 01 Sections "General Requirements" and "Environmental, Safety, and Health Procedures" .  In those cases where excavations are shored, soils at the Lawrence Berkeley National Laboratory shall be considered to meet the OSHA definition of "Type C" soil for the purposes of shoring design, unless evaluated to be other than "Type C" by a enginer licensed in the State of California.

        - Excavation cut not to interfere with normal 45 degree bearing splay of foundations.

        - Grade top perimeter of excavation to prevent surface water from draining into excavation.

        - Hand trim excavation as needed.  Remove loose matter.

        - Remove lumped subsoil, boulders, and rock.

        - Notify the Project Manager promptly in writing of unexpected subsurface conditions before such conditions are disturbed and stop affected work in area until notified to resume work.

        - Correct unauthorized excavation at no extra cost to the University.

        - Correct areas over-excavated by error with [lean concrete with a minimum 28 day compressive strength of 1,500 psi (10.4 MPa)] [or] [in accordance with Division 31 Section "Backfilling"] as directed by the University.

        - Stockpile excavated material in area designated on site and remove excess material not being reused, from site. Cover stockpiled material to protect from rain. Take preventive measures to ensure that water containing soil from excavations or stockpiles does not enter storm drains.

      - FIELD QUALITY CONTROL

        - Field inspection will be performed under provisions of Division 01 Section "Special Provisions."

        - Do not conceal bearing surfaces until inspected by the University’s Inspector.

      - PROTECTION

        - Protect excavations by underpinning, sloping or shoring, as required to ensure life safety and protect property.  Comply with applicable laws and standards.

        - Protect the bottom of excavations and soil within the 45 degree bearing splay of existing foundations.

        - Provide erosion control measures in accordance with Division 31 Section "Slope Protection ".

END OF SECTION 312316