SECTION 321723 – PAVEMENT MARKINGS

- GENERAL

      - SUMMARY

        - Related Documents:

          - Drawings and general provisions of the Subcontract apply to this Section.

          - Review these documents for coordination with additional requirements and information that apply to work under this Section.

        - Section Includes:

          - Pavement striping and markings.

          - Signs.

          - Pavement markers.

          - Parking bumpers.

        - Related Sections:

          - Division 01 Section "General Requirements."

          - Division 01 Section "Special Procedures."

          - Division 32 Section "Asphalt Paving".

          - Division 32 Section "Concrete Paving".

      - REFERENCES

        - General:

          - The following documents form part of the Specifications to the extent stated. Where differences exist between codes and standards, the one affording the greatest protection shall apply.

          - Unless otherwise noted, the referenced standard edition is the current one at the time of commencement of the Work.

          - Refer to Division 01 Section "General Requirements" for the list of applicable regulatory requirements.

State of California, Department of Transportation (CALTRANS)

        - State of California - California Department of Transportation (CALTRANS):

          - Standard Specifications:

            - Sec 56.  Signs

            - Sec 85.  Pavement Markers

            - Sec 94.  Asphaltic Emulsions

- PRODUCTS

      - PAINT

        - White Traffic Paint: DuPont #LF32M30P, Hawkins-Hawkins Co. #V10-31, or equal.

        - Yellow Traffic Paint: DuPont #112-8049, Hawkins-Hawkins Co. #V10-32, or equal.

        - Blue Paint (Handicapped Areas):  DuPont "Precaution Blue" #326Y23665, Hawkins-Hawkins Co. #V10C-1865, or equal.

        - Red Curb Paint:  DuPont #LC64M30P, Hawkins-Hawkins Co. V10C-23, or equal.

      - SIGNS

        - Signs:

          - <Insert product name here>, or equal.

          - International Handicap Sign and Symbol: <Insert product name here>.

        - Substitutions:  Under provisions of Division 01 Section "General Requirements."

      - PAVEMENT MARKERS

        - Amber Bi-Directional Pavement Markers: Reflective type;   Hawkins-Hawkins Co. #V16C-D, Westway, or equal.

        - Amber Mono-Directional Pavement Markers: Reflective type;   Hawkins-Hawkins Co. #V16C-H, Zumar model 88B type H, or equal.

        - Clear Pavement Markers:  Center-mount plastic reflectors, 3-1/4 inch diameter, type GM-75;   Safeway Sign Co., Western Highway Products, or equal.

      - PARKING BUMPERS

        - Three-foot (0.9 m) sections bumper block; Christy Concrete Products No. M20W6BB, Bertelson Concrete Products, or equal.

          - Type: As indicated on the drawings, with steel reinforcement throughout.

- EXECUTION

      - STRIPING AND MARKINGS

        - Mark and stripe uniformly in design, position, and application.

        - Do not apply paint to the paved surface within 24 hours after paving.

        - Apply two coats of paint in accordance with manufacturer's directions.  Protect adjacent surfaces against splatter or stains.

      - SIGNS AND GUIDE MARKERS

        - Install signs and guide markers at locations indicated on the Drawings in accordance with CALTRANS Section 56.

      - PAVEMENT MARKERS

        - Install pavement markers at locations indicated on the Drawings in accordance with CALTRANS Section 85.

      - PARKING BUMPERS

        - Set and level bumpers on pavement with an adhesive consisting of asphaltic emulsion, and anchor to the pavement with two No. 5  by 2-foot  (0.6 m) deformed bars.  Apply the asphaltic emulsion in accordance with CALTRANS Section 94-1.02.

END OF SECTION 321723